COMMAND List

| PROJECT NAME | sAIL Standar |
| --- | --- |

# OEM COMMAND

  - Get BMC Firmware Version

NetFN: 3Ah

Command Code: 33 h

Description:

| Message | Byte | Data Field |
| --- | --- | --- |
| Request | - | - |
| Response | 1 | Completion Code 00h Success |
|  | 2:14 | BMC firmware version ASCII string, LS Byte first. |

  - Get BIOS Version

NetFN: 3Ch

Command Code: 03 h

Description:

| Message | Byte | Data Field |
| --- | --- | --- |
| Request | - | - |
| Response | 1 | Completion Code 00h Success |
|  | 2:9 | Reporting BIOS Version |

  - Set Fan Mode

NetFN: 3ch

Command Code: 3E h

Description:

| Message | Byte | Data Field |
| --- | --- | --- |
| Request | 1 | MODE 00h Full speed: set fans running full speed 01h Optimal speed: set all fans with optimal speed. |
| Response | 1 | Completion Code 00h Success |

  - Get Fan Mode

NetFN: 3ch

Command Code: 3F h

Description:

| Message | Byte | Data Field |
| --- | --- | --- |
| Request | - | - |
| Response | 1 | Completion Code 00h Success |
|  | 2 | MODE 00h Full speed 01h Optimal speed |